SUPPLEMENTARY MATERIAL

Exact Analytical Modeling of a Solar-Ventilated Modernized Qanat for Atmospheric Water Harvesting: Decoupling Proof and Closed-Form Solutions for Chabahar's Humid Climate

Table of Contents

S1. Boundary Conditions Specification

S2. Derived Thermodynamic Properties at Inlet Conditions

S3. Geometric Parameters of Finned Tunnel

S4. Flow Properties and Dimensionless Numbers

S5. Decay Constants and Penetration Depths

S6. Temperature Profile Along Tunnel Length

S7. Humidity Ratio Profile Along Tunnel Length

S8. Local Condensation Rate Distribution

S9. Cumulative Condensate Yield Distribution

S10. System Performance Summary

Table S1: Complete Boundary Conditions

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Tunnel length | L | 500 | m |
| Internal diameter | D | 2.0 | m |
| Depth below ground surface | z | 20 | m |
| Fin height | Hf | 0.1 | m |
| Fin spacing | Sf | 0.3 | m |
| Surface enhancement factor | SF | 3.0 | — |
| Fin efficiency (aluminum) | ηf | 0.95 | — |
| Overall surface efficiency | ηo | 0.967 | — |
| Airflow velocity | v | 1.5 | m/s |
| Inlet air temperature | Tin | 40 | ℃ |
| Inlet relative humidity | RHin | 92 | % |
| Ground/wall temperature | Tw | 28.5 | ℃ |
| Atmospheric pressure | Patm | 101.325 | kPa |

Table S- 1

Table S2: Derived Thermodynamic Properties at Inlet Conditions

| Property | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Saturation pressure at Tin=40°C | Psat,in | 7380 | Pa |
| Partial vapor pressure at inlet | Pv,in | 6790 | Pa |
| Inlet humidity ratio | Win | 0.0445 | kg/kg |
| Inlet humidity ratio | Win | 44.5 | g/kg |
| Saturation pressure at Tw=28.5°C | Psat,w | 3860 | Pa |
| Wall saturation humidity ratio | Ww | 0.0247 | kg/kg |
| Wall saturation humidity ratio | Ww | 24.7 | g/kg |
| Dry air density at inlet | ρdry | 1.127 | kg/m³ |
| Cross-sectional area | Ac | 3.142 | m² |
| Mass flow rate of dry air | ma | 5.31 | kg/s |

Table S- 2

Table S3: Geometric Parameters of Finned Tunnel

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Total perimeter (with fins) | Ptot | 18.85 | m |
| Effective perimeter | Peff | 18.23 | m |
| Hydraulic diameter | Dh | 0.667 | m |

Table S- 3

Table S4: Flow Properties and Dimensionless Numbers

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Reynolds number | Re | 5.97×104 | — |
| Flow regime | — | Turbulent | — |
| Nusselt number | Nu | 139.2 | — |
| Convective heat transfer coefficient | hc | 6.18 | W/(m²·K) |
| Convective mass transfer coefficient | hm | 5.44×10-3 | m/s |

Table S- 4

Table S5: Decay Constants and Penetration Depths

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Thermal decay constant | λh | 0.0211 | m⁻¹ |
| Mass decay constant | λm | 0.0187 | m⁻¹ |
| Thermal penetration depth (99% equilibration) | δh | 218 | m |
| Mass penetration depth (99% equilibration) | δm | 246 | m |

Table S- 5

Table S6: Temperature Profile Along Tunnel Length

| Distance x[m] | Temperature Ta(x)[°C] | Percentage of Total Cooling |
| --- | --- | --- |
| 0 | 40.00 | 0% |
| 50 | 32.16 | 68.3% |
| 100 | 29.66 | 89.9% |
| 200 | 28.62 | 98.9% |
| 250 | 28.54 | 99.7% |
| 500 | 28.50 | 100.0% |

Table S- 6

Table S7: Humidity Ratio Profile Along Tunnel Length

| Distance x[m] | Humidity Ratio Wa(x)[g/kg] | Percentage of Total Condensation |
| --- | --- | --- |
| 0 | 44.50 | 0% |
| 50 | 31.91 | 63.6% |
| 100 | 27.33 | 86.7% |
| 200 | 25.05 | 98.3% |
| 250 | 24.83 | 99.4% |
| 500 | 24.70 | 100.0% |

Table S- 7

Table S8: Local Condensation Rate Distribution

| Distance x[m] | Local Rate dmcond/dx[g/(s·m)] | Percentage Drop from Inlet |
| --- | --- | --- |
| 0 | 1.98 | 0% |
| 50 | 0.72 | 63.6% |
| 100 | 0.26 | 86.9% |
| 200 | 0.035 | 98.2% |
| 250 | 0.013 | 99.3% |
| 500 | 8.1×10-5 | 99.996% |

Table S- 8

Table S9: Cumulative Condensate Yield Distribution

| Distance x[m] | Cumulative Yield [kg/s] | Percentage of Total Yield |
| --- | --- | --- |
| 0 | 0.000 | 0% |
| 100 | 0.0851 | 86.7% |
| 200 | 0.0965 | 98.3% |
| 250 | 0.0976 | 99.4% |
| 500 | 0.0982 | 100.0% |

Table S- 9

Table S10: System Performance Summary

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Exit air temperature | Tout | 28.50 | °C |
| Exit humidity ratio | Wout | 24.70 | g/kg |
| Total condensate mass flow rate | mcond | 0.0982 | kg/s |
| Hourly condensate yield | Vhour | 378 | L/h |
| Monthly condensate yield (12 h/day) | Vmonth | 136 | m³/month |
| Pressure drop | ΔP | 20.92 | Pa |
| Fan power | Wfan | 140.9 | W |
| Specific energy consumption | SEC | 0.37 | Wh/L |
| Required PV panel area | APV,design | 1.54 | m² |
| Required battery capacity | Ebatt | 2.0 | kWh |

Table S- 10